|  | ASIA-PACIFIC TELECOMMUNITY |  |
| --- | --- | --- |
|  | 2nd Meeting of SATRC Working Group on Spectrum in SAP-IV | Document WGSPEC-02/INP-11 |
|  | 11-12 March 2014, Tehran, Iran | # 11 March 2014 |

Nepal Telecommunications Authority

PROGRESS ON WORK ITEM: STUDY OF REGIONAL REQUIREMENTS AND AVAILABILITY OF SPECTRUM FOR MOBILE BROADBAND

INTRODUCTION

## 1.1 Background

The growth of mobile broadband is tremendously increasing nowadays because of its consumer preferred applications including voice, video, and data with mobility, while the market of fixed broadband is not increasing significantly. Aside from the practical benefits of broadband, such as greatly enhanced ease of accessing and providing data-rich content, numerous studies have documented the positive relationship between broadband access and national prosperity. A World Bank study emphasized the importance of broadband penetration for developing economies having concluded that every 10 per cent increase in broadband penetration provides a 1.38 per cent increase in GDP.

The telecommunication industry has been focused to the development of such mobile broadband solutions in reliable and affordable manner. The market of mobile broadband is tremendously increasing compared to that of the fixed broadband solutions. High speed and mobility are two dimensions for mobile broadband. Initially, broadband  started without mobility and it emerged during the period of different evolutions. Traditionally, mobility emerged with cellular systems for voice communications. Success of broadband wireless networks is influenced by many factors like equipment cost, infrastructure costs, customer penetration, availability of spectrum and many more. Technical aspects, business aspects, and service aspects of the emerging mobile broadband technologies are the key domains to be considered. System architecture, spectrum, mobility, multiple access techniques, multiple antenna techniques, capacity and data rate are key technical parameters which distinguishes the performance of the technologies. CAPEX, OPEX, subscriber and revenue is very much vital regarding the business model. Study of deployment cost, revenue generating options, sustainability issue, and regulatory issue gives idea of the financial model of these technologies.

Spectrum is one of the most valuable and finite sources and must thus be allocated in such a way that ensures maximum efficiency both technically and economically.

Mobile Internet, Mobile Banking, Mobile Cash Exchange, Online Gaming, Social Packs ( Facebook, Twitter, Linkedin),Email (Outlook, Gmail),Chat (Facebook, Gmail, Mig33, Yahoo, MSN ), Messaging services (Whatsapp, Line, Viber, Wechat, Skype),News (ekantipur, Onlinekhabar), Streaming( Youtube, Daily Motion),Online TV( Starsports, Starplus), Online Games( Temple Run, Subway surf, Candy Crush), 2P based services (Torrent),Movie and songs downloads etc have gained popularity among large number of users in mobile network in past few years .  The different services or applications that generate large amount of mobile data traffic in the mobile network includes mobile Internet, online Gaming, social Packs, streaming, news based services ,URL based services (Facebook pack),IPTV (Mobile TV),ecommerce, video conferencing, online Games, messaging services, cloud based services, Machine2Machine applications, telemedicine,mobile Money applications,streaming services, remote Education,chat based services,email services, news based services etc.

It is necessary to take into account the demand of services, effective and efficient technical and operational framework with optimal spectral efficiency. The calculation has to consider the number of base stations within a given market and scope for future site densification. The spectrum calculation model has to also consider the traffic load per site applicable for both the data and voice.  Global mobile data traffic growth Based on global statistics the number of wireless broadband subscribers has exceeded the number of fixed broadband subscribers and will continue on an explosive growth path as per current growth estimates.

More than 90 percent of the world’s population is under the coverage of mobile networks, and the first and the only way to access the internet, for many people in the developing countries, is IMT network and IMT network traffic is quickly increasing in accordance with the new services and applications development based on the broadband. Therefore the required bandwidth for IMT would be also increased day-to-day and numerous appeals have been sent to the regulators by mobile operators for increasing the required bandwidth. Now, one of the main and the important concerns for the world regulators is forecasting the required spectrum for IMT networks in the future so that they can supply the required spectrum of IMT network based on the market's requirement through policing and planning accurately and allocating the suitable frequency sub-bands. In this regard, The agenda items of WRC-15 AI 1.1 dealing with spectrum matters for IMT  is to consider additional spectrum allocations to the mobile service on a primary basis and identification of additional frequency bands for International Mobile Telecommunications (IMT) and related regulatory provisions, to facilitate the development of terrestrial mobile broadband applications, in accordance with Resolution 233 (WRC-12);

. As regards, the first step for making decision about Agenda Item 1.1 is estimating the required spectrum for IMT and the countries membered in SATRC shall supply and support the required spectrum for IMT in order to plan and policy accurately and also declare the required spectrum of countries membered in SATRC to regional and international councils, this document has been prepared under Work Item S.2 entitled "Study of Regional Requirements and Availability of Spectrum for Wireless Broadband" and requested from the countries membered in SATRC to estimate the required spectrum of the countries in 2020 and after gathering the responses, a report will be prepared entitled "SATRC Report on spectrum requirements for IMT

The mobile broadband subscriptions growth forcasted by Ericssion is shown below by 2015.

According to the Cisco Visual Networking Index Report 2012, Global Mobile Data Traffic Forecast overall mobile data traffic is expected to grow to 10.8 Exabytes (1 Exabyte=10 18 bytes) per month by 2016. The growth tendency is shown in fig given below:

Demand on spectrum is increasing rapidly due to user demand as well as innovation of various wireless/mobile broadband technologies. Wireless broadband technologies have now become the major tools to bridge the digital divide in developing countries. There is a need for a focused study about the requirements of spectrum for appropriate wireless broadband technologies in SATRC countries.

This report focuses Study of Regional Requirements and Availability of Spectrum for Mobile Broadband technologies. The purpose of this study is to study the regional requirements for spectrum and the availability of those bands in SATRC countries.

1.2 Objectives

The objective is to carry out an assessment on requirement analysis with a view to facilitate deployment of wireless broadband technologies taking into account convergence trends and provide recommendations for future spectrum requirement in SATRC member countries .

It is also a necessity for the assessment on availability of spectrum of mobile broadband in the SATRC countries in order to create an environment for the availability of sufficient mobile broadband spectrum for the development and expansion of telecommunication services in order to boost the social life for the benefit of the public in the member countries.

1.3 Scope

| Work Item | S2. Study of Regional Requirements and Availability of Spectrum for Wireless Broadband |
| --- | --- |
| Responsible Working Groups | WG Spectrum |
| Output | Study Report/Guideline |
| Background and Purpose | - Demand on spectrum is increasing rapidly due to user demand as well as innovation of various wireless/mobile broadband technologies. Wireless broadband technologies have now become the major tools to bridge the digital divide in developing countries. However, there is a need for a focused study about the requirements of spectrum for appropriate wireless broadband technologies in SATRC countries with special focus on IMT applications. - Purpose of this study would be to study the regional requirements for spectrum and the availability of those bands in SATRC countries |
| Scope | - To survey the requirement of spectrum for Wireless Broadband in SATRC countries - To identify the bands for Wireless Broadband - Study the use of those bands in SATRC countries - Suggest harmonized approach to make the spectrum bands available for Wireless Broadband |
| Time Frame | Total study period would be approximately 1 year |
| Utilization of Output | Members can use the output as reference while allocating spectrum for Wireless Broadband |

1.4. Methodology

The study was carried out using the primary and/or secondary data during the preparation of this report. During the research study, a questionnaire was prepared and sent to the SATRC member countries and the response from the respective expert/countries is taken into the consideration. Hoever the response from all the SATRC member countries was not received. The data/information related to the spectrum requirment available in the web/internet is also taken into account. The different data/information from the international best practices/ white papers/publications is also taken into account for the analysis in the report as secondary data/information.

2.   Review of Global Scenario

  - Mobile Broadband Spectrum Requirement/Global Scenario- Prediction & Gap

Mobile broadband spectrum requirement will increase sharply due to m-applications including M2M activities and introduction of many new services. Increased demand is attributed to an overall increase in the volume of data downloaded as a result of greater consumer use and machine to machine interactions. It is nneeded to upgrade their network with advanced technologies to meet the market demand.

The frequency bands identified for IMT services

The frequency bands identified for IMT services (that accommodate all the technologies)through different ITU WRCs are mentioned hereunder with distinct colors.

Band (MHz) 			     Radio Regulation Footnotes identifying the band for IMT

450-470                                                               5.286AA

694-790/698-806*/790-862*806 – 960                 ADD 5.3XX,MOD 5.313A, MOD 5.317A

1 710–1 885, 1 885-2 025                                 5.384A, 5.388

2 110-2 200 					   5.388

2 300-2 400 					   5.384A

2 500-2 690 					   5.384A

3 400-3 600					   MOD 5.430A, 5.432A, 5.432B, 5.433A

Note:

World Radio Conference – 1992

World Administrative Radio Conference –2000

World Administrative Radio Conference – 2007

World Administrative Radio Conference – 2012

(Allocation shall enter into force on 1 January 2013)

In 2007, ITU- R estimated that for mobile broadband applications, between 1280 MHz to 1720 MHz spectrum may be required by 2020. The spectrum demand forecast for mobile broadband (2010-2020) from the ITU-R Report M.2078 (2007) is shown below.

The FCC announced in 2010 that an additional 500 MHz of spectrum would be needed for mobile broadband technologies by 2020. Some international regulators have determined that up to 500 MHz of spectrum beyond that already available may be required to meet the demand for mobile broadband services by 2015. ACMA estimates that an additional 160 to 700 MHz of spectrum will be required by 2020. It has been estimated that additional 150 MHz to 1720 MHz spectrum may be required by 2020.

The European Union (EU) released its Digital Agenda for Europe: 2010–2020 in May 2010 setting out the following objectives:

  - ensure broadband coverage of all EU citizens by 2013

  - offer broadband coverage at 30 Mbps or more for at least half of EU households  by 2020.

Some experts believe that since the life time of a technology is around 15-20 years, it is more pragmatic to concentrate on a 5 year time frame.

Figure below shows the comparison between the estimated required, ITU identified and regionally available spectrum.

Fig: Comparison of the amount of the estimated required, global identified and regional available   	spectrum (source: ITU-R M.2078 & UMTS Jan. 2012)

During the preparation for WRC-15, spectrum requirement estimation is ongoing in ITU Working Party 5D. The estimated requirement is in total around 1800MHz (using the higher requirements setting). Compared with the 1172MHz already identified, it is clear that more than 500MHz of additional spectrum is needed.

According to the mobile global data traffic estimation summarized in ITU M.2443 (00/11), overall mobile data traffic averagely grow 8 times in 2015 over 2011. Visioning the future year 2020, the traffic is 500-1000 times today’s traffic, driven by the demand for mobile broadband for anything, anytime from anywhere.

The explosively increasing mobile traffic is not distributed evenly over the whole network and more than 80% of the traffic comes from hotspots or indoor areas, based on the analysis from Informa Telecoms & Media ( source: Mobile Broadband access at home: Informa Telecoms & Media) . It is also forecasted that mobile video will be the dominant service in the near future and it is shown that about 70% of mobile services will be video in 2016 based on the prediction of mobile traffic share from Cisco (Source: cisco, cisco visual Networking Index, Global Mobile Data Traffic Forecast Update, 2011-2016).

To meet the explosive traffic demands and higher performance expectation, the heterogeneous network or HetNet is becoming the network topology of the future. As mobile traffic increases and mobile connection speeds increase for anything, anytime from anywhere, more spectrum in the low and low-to-mid bands is needed to provide the coverage and capacity. The mid-to-high band is much more important than ever before, to provide high performance, and also to provide capacity boosting for the urban environment, especially hotspot and indoor areas.

2.2 Spectrum Requirement Calculation by Different International standard Bodies

2.2.1 ITU

ITU has recommended a Methodology for calculation of spectrum requirements for the terrestrial component of International Mobile Telecommunications in April 2013. ITU-R M.1768-1 describes a methodology for the calculation of terrestrial spectrum requirement estimation for IMT. It provides a systematic approach that incorporates service categories (a combination of service type and traffic class), service environments (a combination of service usage pattern and teledensity), radio environments, market data analysis and traffic estimation by using these categories and environments, traffic distribution among radio access technique groups (RATGs), required system capacity calculation and resultant spectrum requirement determination. The methodology is applicable to packet switch-based traffic and can accommodate multiple services. It can also accommodate circuit switched emulation traffic using a reservation based concept.

The ITU Radiocommunication Assembly has considered that the radio access technique groups (RATGs) that are appropriate for IMT may have different channel bandwidth requirements, and hence varying impact on the basic frequency usage possibilities; and also the service functionalities in fixed, mobile and broadcasting networks are increasingly converging and interworking. The methodology for calculation of spectrum requirements for IMT has considered the followings:

- accommodate the complex mixture of services that will require differing bandwidths and quality of service, and significantly higher bit rates than IMT-2000

- use market data which is practical to collect as input to the traffic forecasts;

- have the flexibility to handle both emerging technologies and enhancements to IMT;

- take account of factors for practical network implementations;

- produce results in a manner that is easily understandable and credible;

- be implementable and verifiable within the available time-scales;

- be suitable to be used during ITU-R meetings in terms of the computing facilities needed and the time required to perform an analysis;

- be no more complex than is justified by the uncertainty of the input data;

- take into account improvements in spectrum efficiency due to the advances in technologies employed by enhancements to IMT

The calculation methodology has recognized

- that the majority of the traffic has changed from speech-oriented communications to multimedia communications;

- that networks and systems are designed to economically transfer packet data;

- that services become more diverse and it will be less valid to consider simple peak traffic values that will apply across different environments, geographic areas and time,

- The Prerequisite information for application of the methodology consists of Forecast on services and market, and Technical considerations.

Approach for spectrum calculation

The technical process of estimating spectrum requirements for mobile communications has to be

based on four essential issues:

– Definition of services

– Market expectations

– Technical and operational framework

– Spectrum calculation algorithm

- The spectrum claculation method has considered service environments as a function of combination of service usage pattern and teledensity. It has also considered the Traffic classes  from the user perspective: conversational class of service, interactive class of service, streaming class of service, and background class of service. The main distinguishing factor between these classes is how delay-sensitive the application is: conversational class refers to applications which are very delay-sensitive while background class is the most delay-insensitive QoS class. Service environments represent common service usage and volume conditions.

Teledensity, Radio Environment, RATGs, and analysis of the collected market data are also the key dimensions to be considered for the calculatin of spectrum requirment.

Each teledensity parameter is characterized by population density and communication device density. Teledensity is categorized into the followings: Dense urban, Suburban, Rural.Radio environments (REs) are defined by the cell layers in a network consisting of hierarchical cell layers, i.e. macro, micro, pico and hot-spot cells. The cell area of macro cell is made teledensity dependent in spectrum requirement calculations.  Following four types of RATGs are identified.

- Group 1: Pre-IMT systems, IMT-2000 and its enhancements

- Group 2: IMT-Advanced systems as described in Recommendation ITU-R M.2012.

- Group 3: Existing radio LANs and their enhancements.

- Group 4: Digital mobile broadcasting systems and their enhancements.

Analysis of the collected market data:

Collection of market data:

The  following items are relavant in order to survey future market and application

trends:

– services and market survey for existing mobile services;

– key market parameters;

– service and market forecast for IMT, including:

• service issues;

• market issues;

• preliminary traffic forecast;

• related information;

– service and market forecast for other radio systems;

– driving forces of the future market; and

– any other views on future services.

Data analysis:

Application, Service Market attribute parameters, Traffic attribute parameters are the key terminologies  for market data analysis:

General process for the market data analysis

The ITU Recommendation presents the methodology for calculating the spectrum requirements for the further development of IMT. The methodology accommodates a complex mixture of services from market studies with service categories having different traffic volumes and QoS constraints. The methodology takes into account the time-varying and regionally-varying nature of traffic. The methodology applies a technology neutral approach to handle emerging as well as established systems using the RATG approach with a limited set of radio parameters. The four RATGs considered cover all relevant radio access technologies. The methodology distributes traffic to different RATGs and radio environments using technical and market related information. For RATG3 and RATG4 no spectrum requirements are calculated. For the traffic distributed to RATG1 and RATG2 the methodology transforms the traffic volumes from market studies into capacity requirements using separate algorithms for packet-switched and circuit switched (reservation based) service categories and takes into account the gain in multiplexing packet services with different QoS characteristics. The methodology transforms capacity requirements into spectrum requirements using spectral efficiency values. The methodology considers practical network deployments to adjust the spectrum requirements and calculates the aggregate spectrum requirements for further development of IMT.

Total spectrum requirements in the year 2020 calculated by ITU-R WP 5D

|  | Spectrum requirements |  | Total spectrum requirements |
| --- | --- | --- | --- |
|  | for RATG 1 | for RATG 2 |  |
| Lower user density settings | 440 MHz | 900 MHz | 1 340 MHz |
| Higher user density settings | 540 MHz | 1 420 MHz | 1 960 MHz |

During WRC-07, countries and administrations recognized the need for additional IMT spectrum and identified new bands for IMT. Though significant progress has been achieved since WRC-07, the spectrum currently identified for IMT networks is still far below the amount recommended in ITU-R Report M.2078.

2.2.2 GSMA

GSMA has carried out study for the estimation of spectrum BW required by 2020 based upon its calculation model.  The model allows information to be input in a number of different ways but the basic calculation is straightforward and given by:

S= (T/N x M ) / C

Where S is Spectrum (in MHz), T is Total Traffic (in Mbps), and N is Number of Macro Cell sites, M is Load Multiplier for highly congested sites, and C is capacity of sites (in Mbps/MHz).

The model currently takes forecasts of regional monthly mobile data traffic (in millions of GB) from Cisco and monthly voice minutes of usage from Wireless Intelligence. These are converted to average busy hour site traffic (T) assuming:

- 15% of the daily data traffic occurs in the average site data busy hour

- 8.5% of the daily voice traffic occurs in the average site data busy hour

- Voice is converted to data at an equivalent rate of 21 kbps.

The calculation has considered the number of base stations within in a given market and scope for future site densification. It has also considerd the spectral efficiency of 0.3 bps/Hz in 2012 and with the estimation of 1.96 bps/Hz by 2020. The capacity of each site is determined by basic spectral efficiency for the technology, the number of sectors per site, signalling overhead and a quality margin that will ensure good quality and low delay. The spectrum calculation model has also considered the traffic load per site applicable for both the data and voice.  The calculation of the estimation results for about 2 GHz spectrum Bandwidth by 2020 to cater the mobile broadband appliations/services.

2.2.3 UMTS Forum

UMTS has published a White Paper from the UMTS Forum: Spectrum for future development of IMT-2000 and IMT-Advanced January 2012.

Data traffic on mobile networks continues to grow, demonstrating the immense global importance of mobile broadband as a key socio-economic enabler. Mobile broadband literally changes the way we live by offering services that were previously not available. The demand for mobile cloud services is expected to grow exponentially since the users are increasingly adopting more services that are required to be accessible. Multimedia services captured on mobile devices will overwhelmingly carry the greatest cloud computing and storage demand. It is expected that e-health, e-education and other e-government services will also be accessed by mobile devices, which will contribute to improvements in social welfare.

An industry expertise in traffic prediction and spectrum requirements:

In 2005, UMTS Forum Report 37 (Magic Mobile Future 2010-2020) concluded that mobile traffic would increase by a factor of 23 between the years 2012 and 2020. In 2006 UMTS Forum Report 40 (Development of spectrum requirement forecasts for IMT-2000 and systems beyond IMT-2000 (IMT-Advanced)) analysed spectrum demand, concluding that a total of around 1600 MHz of spectrum is required for IMT networks by 2020, including the spectrum already allocated at WARC-92 and WRC-2000.

Updated UMTS Forum traffic predictions:

The UMTS Forum Report 44, Mobile traffic forecasts 2010-2020 states that: “the total worldwide mobile traffic will reach more than 127 Exabytes in 2020, representing an x33 increase compared with 2010 figure. Significantly, at least 80% of the traffic volume remains generated by users, leading to large variations of the total mobile traffic both in terms of time and space variations of traffic. Future mobile networks must be designed to cope with such variation of traffic and uneven traffic distribution, while at the same time maintaining a permanent and extensive geographical coverage in order to provide continuity of service to customers. In 2020, daily traffic per Mobile Broadband subscription in the representative Western European country will stand at 294 MB and at 503 MB for dongles [representing a x67 increase compared with 2010 figures].

2.2.4 Huawei

In Feb 2013 Huawei, the established international manufacturer, has published a study report with the conclusions regarding the spectrum requirement for the mobile broadband applications/services by 2020. The study report has suggested for WRC15 to identify the spectrum in the spectrum bands of 470-694 MHz, 694-790 MHz, parts of L band, the band around 2GHz, parts of 3600-4200MHz and 4400-4990MHz.  It has also indicated that besides acquiring new spectrum for IMT, the efficient use of existing spectrum is another way to promote the development of the wireless industry. Small cell deployments and the allocation of appropriate high-frequency spectrum for hotspot applications, supplementary downlink spectrum, carrier aggregation and LTE roaming bands are explored as methods to utilize spectrum better. The study has drawn the following conclusions:

Identify at least additional 500MHz (in the 400MHz – 6GHz range) spectrum Bandwidth at WRC-15 taking into the consideration of the followings:

- Targeting global harmonization to the benefit of economies of scale

- Targeting assignments of at least 100MHz contiguous bandwidth for IMT

- Driven by the well recognized socio-economic value of the mobile broadband

application

- Administrations need to take efforts in reducing the time that is currently  separating the ITU-R identification from the actual spectrum assignments at national level

- 3.5GHz (3400-3600) as one of the important bands of global spectrum for small cell enhancement

Spectrum shall be efficiently utilized taking into account the followings:

- based on CA solution, and mixed TDD+FDD CA as one of future trends

- Candidate bands combination for LTE FDD terminal roaming at least include

- 1800MHz, E850MHz, APT700MHz and US 700MHz

- Inter-operators’ network synchronization based on over-the-air solution proposed for TDD networks

It has identified the possible candidate band for IMT under WRC-15 Agenda Item 1.1 with the following bands to target for the allocation in WRC'15.

- Low candidate bands( <1 GHz): 470-694 MHz with cooperation with the broadcasting industry( e.g TV, PMSE)

- Low to Mid Candidate Bands (1GHz-3 GHz): 1350-1525 MHz, 2700-2900 MHz in coperation with the existing radar users.

- Mid to High candidate bands( 3-6 GHz): 3.4- 4.2 GHz and 4.4-4.99 GHz band

2.3 Spectrum Requirements forecast by the different Administrators

The number of sites per operator indicating the rate of densification results to give different values of spectrum requirment in urban, suburban and rural areas.  The required number of sites increases with the availabe/usage spectrum bandwidth and vice versa. The spectrum requirements forecast by the different administrators is shown in the figure given below:

| Administrator | Information Source | Traffic forecast | Baseline bandwidth for IMT | Additional Spectrum Requirement |
| --- | --- | --- | --- | --- |
| Europe | European Union radio Spectrum Policy Programme(RSPP) |  |  | Y2015: 1200 MHz |
| USA | FCC National Broadband Plan | 35 times increase in traffic from 2009 | Y2009 allocated:547 MHz | Y2014: 300 MHz for mobile broadband Y2020: 500 MHz for Mobile and fixed broadband |
| Canada | Globe Mobile Broadband Forum 2012 |  | Y2014 Allocated: 553 MHz | Y2015: 300-500MHz Y2022: 400-600 MHz |
| Australia | ACMA paper towards 2020-Future Spectrum Requirements for Mobile Broadband | 30 times increase in traffic from 2007 to 2014 | Y2012 allocated and planned:840 MHz | Y2015: 150 MHz Y2020: 300 MHz |
| Japan | AWG Workshop for Future IMT(AWG-13/INP-136) | Growth rate of traffic is increasing to more than 100% per year | Y2012 allocated:500 MHz | Y2015: Over 300 MHz Year 2020: total over 1000 MHz |
| China | ITU-R WP5D#15( document 5D/256) | Around 600 times increase in traffic from 2010 to 2020 | 690 MHz | Y2020: 800-1100 MHz |

Source:  Whitepaper on Spectrum Feb, 2013, by Huawei

UK

A total spectrum of 2074 MHz will be required in 2020, assuming the number of macro-sites per operator to be 16,370

Brazil

A total spectrum of 2080 MHz will be required in 2020 assuming to a total of 22,850 macro-sites per operator 1

China

A total spectrum of 1844 MHz will be required in 2020 assuming a total of about 472,900 macro-sites per operator 1

USA

A total spectrum of 1939 MHz will be required in 2020 assuming a total of about 84,420 macro-sites for a national operator 1

As is being discussed in ITU-R WP5D, more than 500 MHz of additional spectrum is needed for the year 2020, distributed in three band ranges – low band (<1GHz), mid-to-high band (1-3GHz) and high band (3-6GHz), to support the explosive traffic increase and higher performance expectation.

India

India is one of the fastest growing telecom markets in the world.  The unprecedented increase in teledensity and sharp decline in tariffs in the Indian telecom sector have contributed significantly to the country’s economic growth. Besides contributing to about 3% to India’s GDP, Telecommunications, along with Information Technology, has greatly accelerated the growth of the economic and social sectors. Given the continued predominant role of wireless technologies in delivery of services in ICT sector, NTP 2012 incorporates framework for increasing the availability of spectrum for telecom services including triple play services (voice, video and data) for which broadband is the key driver. The objectives of the NTP-2012 include to Increase rural teledensity from the current level of around 39 to 70 by the year 2017 and 100 by the year 2020. It has also envasized to provide affordable and reliable broadband -on -demand by the year 2015 and to achieve 175 million broadband connections by the year 2017 and 600 million by the year 2020 at minimum 2 Mbps download speed and making available higher speeds of at least 100 Mbps on demand.

It has been mentioned to ensure adequate availability of spectrum and its allocation in a transparent manner through market related processes. Make available additional 300 MHz spectrum for IMT services by the year 2017 and another 200 MHz by 2020.

It has envisaged the strategy to make available adequate globally harmonised IMT spectrum in 450 MHz, 700 MHz, 1800 MHz, 1910 MHz, 2.1 GHz, 2.3 GHz, 2.5 GHz, 3.5 GHz bands and other bands to be identified by ITU for commercial mobile services.

Recognizing this growth of IMT services and considering the future requirements, the National Telecom Policy 2012 highlights the need of adequate availability of spectrum with a target of making available additional 300 MHz spectrum for IMT services by the year 2017 and another 200 MHz by 2020.

Japan

In 2010, it was envisazed that by y 2020, Radio spectrum use will have dramatically expanded, both in quality and quantity, and the amount of traffic will be 200 times larger.

The action plan for spectrum reallocation to realize wireless broadband developed by Japan has pointed out the basic policies to secure spectrum towards the years 2015 and 2020, respectively:

–	Secure additional spectrum exceeding 300 MHz bandwidth below 5 GHz frequency ranges for mobile communications systems and wireless sensor network systems by 2015;

–	Secure additional spectrum exceeding 1500 MHz (including the above mentioned 300 MHz) bandwidth for IMT-Advanced systems and for broadband deployment in airplanes, ships, trains, etc.

The MIC also annually updates the “Action Plan for spectrum reallocation” to describe specific actions for a smooth and steady follow up toward the reallocation of the radio spectrum, based on the evaluation results of the survey on actual radio spectrum use. This Action Plan also indicates several target frequency bands for use by mobile communications including IMT.

Australia

The Australlian Communication & Media Authority (ACMA) has undertaken a study and published a study report in May 2011 regarding the Future Spectrum Requirments for Mobile Broadband - towards 2020. From the analysis, the ACMA estimates that an additional 150 MHz of spectrum will be required by 2020. This estimate takes into account the 800 MHz of spectrum already dedicated for operation by mobile communications services; and includes the 150 MHz previously identified by the ACMA as being required by 2015.  It was identified that approximately 1100 MHz of spectrum would be required in the Australian communications environment to support mobile broadband services by 2020.

The spectrum estimation considers possible re-planning activities for the band including ‘refarming’ the digital cellular mobile telephony service segments (890–915 MHz paired with 935–960 MHz), currently planned for GSM technologies, to better facilitate 3G and 4G technologies and provides analysis on the potential for the 850 MHz ‘expansion’ band to be made available for future mobile broadband services.

China

A model of spectrum requirements to meet IMT service in China until 2020 has been developed and published by China Academy of Telecommunication Research of MIIT in Jan 2013. The modelling has considered the spectrum requirements for 2G technologies, 3G technologies and 4G technologies, with the assumption that China commercially launches 4G networks in late 2014. 687 MHz spectrum BW has been allocated for IMT system so fa in China.

IMT services are experiencing considerable growth in China, mainly driven by consumer demand for mobile data. The report has analyzed the sensitivity analysis of the spectrum requirment for the mobile broadband wrt data traffic growth rate, no. of macro base stations, downlink traffic percentage of toatl traffic, and no. of operators. The report indicates that the maximum data traffic from IMT services in 2020 would achieve nearly 200 Mega-Tera-Byte per year, about 600 times of 2011. And even more base sites would be constantly deployed with CAGR of around 13.65% and the average spectrum efficiency of 2020 would be more than 4 times of that of 2011, it still could not comparable to the increase of data traffic, accordingly the spectrum bandwidth requirement. It is forecasted that the cell site growth rate relative to 2011 is 62143% by 2020 to increase mostly the capacity. And the average spectrum efficiency would be 1.61 by 2020 which is 0.43 in 2012 which will be around 4 times higher than that of 2011.

The result demonstrates that around 1800 MHz spectrum is likely required from IMT services by 2020. The total spectrum BW was calculated as the sum of 2G/3G/4G spectrum requirements. Currently China has allocated 687 MHz frequency for IMT systems so that approximately over 1100 MHz spectrum deficit would appear by 2020.

Suitable Frequency Bands under Consideration for the mobile bradband recommended by the China:

- 606-698 MHz

- 1427-1518 MHz

- 1695-1710 MHz

- 2700-2900MHz

- 2900-3100 MHz

- 3100-3300 MHz

- 3300-3400 MHz

- 3600-3700 MHz

- 4400-4500 MHz

- 4500-4800 MHz

- 4800-4990 MHz

- 5350-5470 MHz

- 5850-5925 MHz

- 5925-6425 MHz

Nepal

ITU has published a Wireless broadband masterplan Report for the Federal Democratic Republic of Nepal in October 2012. In the report It was recommended that the government should aim to provide at least total of 442 MHz for mobile services by 2015 in the 700, 900, 1800, 2100 and 2300 MHz bands. In particular, the 900 MHz band should be refarmed on efficiency grounds so that spectrum is utilised on a national basis and that 36 per cent of the 900 MHz spectrum band that is currently only in use by rural providers is made available for reuse.

For the longer term up to 2020, this masterplan prefers an approach of using the overall spectrum requirements in a manner consistent with ITU-R Report M.2078 (2006). In the case of Nepal, on balance, the aggregate target for spectrum to be allocated to wireless services ought to be the smaller market target of 760 MHz of spectrum by 2020 is recommended.

### 2.4 Spectrum in LTE

LTE (Long Term Evolution) promises to deliver very high speed broadband with low latency, flexible spectrum bandwidth, smooth migrations and low cost data services.

WRC-2000 identified three additional bands for terrestrial IMT-2000 including 2500-2690 MHz. In Europe, 140 MHz bandwidth of 2.5 GHz of the of IMT-2000 FDD expansion spectrum was allocated in the beginning of 2008; 2620-2690 MHz for downlink and 2500-2570 MHz for uplink . Additionally up to 50 MHz (2570 MHz-2620 MHz) will be allocated as an unpaired TDD band. The main goal of WRC-07 was to identify additional, worldwide spectrum, to enable global roaming services. For this purpose, WRC-07 identified 450-470MHz and 2300-2400 MHz bands for IMT (both IMT-2000 and IMT- Advanced) on a global basis. WRC-07 has made positive steps towards making spectrum available for future LTE deployments.

WRC-07 identified the bands 698-806 MHz for mobile applications and initiated the process of allocation of broadcasting spectrum in the 698-806 MHz band to mobile applications.  In fact there are different applications in different countries in this band, so the next step would be to identify the cases in different regional countries and gradually refarm the spectrum for mobile applications to the maximum extent in order to achieve the interoperability and seamless global roaming.

### Refarming GSM 900 MHz

The 900 MHz is most harmonized spectrum band available today in wireless telecommunication industry and this band has benefit of increased coverage worldwide. Moreover, 900 MHz offers improved building penetration and especially suited to regions that have a predominantly rural population. Moreover, LTE deployment in the 900 MHz has the benefit to have more capacity and provide operators to deploy an LTE network with greater coverage.

### Deployment Strategies with LTE spectrum

There is a possibility for GSM and WCDMA operator to use their existing bands for deploying LTE as 1800 MHz and 2100 MHz of spectrum can be reused for LTE system. LTE allows operation in the 1.4, 3, 5, 10, 15 and 20 MHz channel sizes. This flexibility and possibilities of choosing different bandwidth let operators to introduce LTE in existing GSM/WCDMA band.

There is also a possibility for GSM and WCDMA operators to introduce LTE in new spectrum like 2.6 GHz, 2.3 GHz or 0.8 GHz. The advantage of introducing new spectrum let the operators to use larger bandwidths. So operators have the choice to introduce LTE in existing band. One of the big advantages of the LTE is that it supports both the smaller and bigger channel bandwidths. There are possibilities for the operators to deploy LTE networks from 1.4 MHz to channel up to 20MHz.

2.5 Key challenges

In order to materially increase wireless broadband penetration, the SATRC member administrations need to devise solutions to successfully address the following challenges including power disruptions, affordability, and policy and regulatory barriers. The key challenges in the SATRC include the followings:

- Freeing the existing bands used by other services rather than IMT

- Implementation of Analogue switchover

- Refarming of existing bands for the purpose of IMT/IMT-Advanced ( Adoption of efficient technologies with the phasing out of the existing 2G services) [3G in 900 & LTE in 1800 MHz bands)

- Lack of appropriate policy and regulatory environment

- Implementatin delay of the the declared policy framework

- Lack of expert in identifying the national requirment and develop the required strategy for the allocation and subsequent assignment

2.6 Best Principles for spectrum management

The best practice guidelines for spectrum management of Mobile Broadband to be considered in SATRC member countries are mentioned hereunder:

- Facilitate the deployment of innovative broadband services and technologies

- Allocate spectrum to the highest value use or uses

- Promote transparent and nondiscriminatory spectrum management policies

- Embrace technology neutrality

- To adopt flexible use measures for wireless broadband services

- Ensure affordability

- Manage spectrum efficiently

- Harmonize regional and international standards and practices

- Enable and encourage spectrum to move to its highest value use or uses

- To the extent possible, promote both certainty and flexibility

2.7 Utilization of key spectrum including below 1 GHz Frequency

Radio spectrum below 1 GHz is optimum for the needs of developing countries, due to the ability to serve larger rural areas from a single cell site compared to spectrum above 2 GHz. The 2007 World Radio Conference made valuable strides in identifying additional spectrum for IMT, both below 1 GHz and above 2 GHz. The concept of identifying spectrum for potential use by IMT, in the ITU Radio Regulations, gives global equipment manufacturers some guidance on the range of frequency bands in which IMT services are likely to be deployed, leading to economies of scale and minimizing product costs.

Given the dominant position of mobile services (which will grow even more so) and wireless broadband in the future, the need to utilise key spectrum below 1 GHz spectrum resource (especially the 700 MHz band) is profound. In this respect, the digital dividend spectrum shall be locked for mobile broadband which will be also in line with Asia Pacific proposal.

The identification “for those administrations wishing to deploy IMT” allows use by other services to which the spectrum is allocated and does not convey any priority for IMT over those other radio-based services.  “IMT-Advanced” provides a global platform on which to build the next generations of mobile services – fast data access, unified messaging and broadband multimedia – in the form of exciting new interactive services and applications.

2.8 Flexible rights of use for key wireless spectrum allocations

Flexible rights of use should be instituted for key wireless spectrum allocations with technology use given that artificial technology limitations may deny operators being able to use efficient and spectrally efficient technology. For example, 3G W-CDMA at 900 MHz is more cost effective than rollout of 3G W-CDMA at 2100 MHz specially in the less densely populated areas.

The spectrum allocation for the mobile broadband shall be in technology neutral basis to facilitate for the deployment of effective and efficient technology in the market which will also provide an early upgrade path to take account of more affordable devices including smartphones and tablets in the market. Satellite services – perhaps with a local cellular or Wi-Fi hotspot can also provide a critical role in addressing digital divide in more remote areas of the country. Facilitating WBB services is best promoted by introducing additional operators  and by the early allocation of sub-1 GHz spectrum especially additional 900 MHz and the allocation of 700 MHz band spectrum (in accordance with the APT 2 x 45 MHz band plan). However, in order to balance spectrum availability with the legitimate public goal of maximising the price of the country’s spectrum resources, it is recommended that a staged approach to the release of new spectrum be adopted.

2.9 Frequency Arrangements for implementation of IMT

Preliminary Draft Revision of Recommendation ITU-R M.1036-3 on Frequency arrangements for implementation of the terrestrial component of IMT in the bands identified for IMT in the Radio Regulations (RR)’ in Annexes 1 to 6, describes the frequency arrangements for implementation of IMT in the bands identified for this service in the ITU RR. The order of the frequency arrangements does not imply any priority. Administrations may implement any of the recommended frequency arrangements to suit their national conditions. Administrations may implement all or part of each frequency arrangement.

It is noted that Administrations may implement other frequency arrangements (for example, arrangements which include different duplex schemes, different FDD/TDD boundaries, etc.) to fulfil their requirements. These administrations should consider geographical neighbouring deployments as well as matters related to achieving economies of scale, facilitating roaming, and measures to minimize interference.

Recommendation ITU-R M.1036-3 dealing with Frequency arrangements for implementation of the terrestrial component of IMT in the bands identified for IMT in the Radio Regulations (RR)’ provides guidance on the selection of transmitting and receiving frequency arrangements for the terrestrial component of IMT systems. The frequency arrangements are recommended from the point of view of enabling the most effective and efficient use of the spectrum to deliver IMT services – while minimizing the impact on other systems or services in these bands – and facilitating the growth of IMT systems.

General considerations regarding technological aspects

- IMT (IMT-2000 and IMT-Advanced) radio interfaces currently include two modes of operation – frequency division duplex (FDD) and time division duplex (TDD);

- There are benefits in the use of both FDD and TDD modes in the same band; however, this usage needs careful consideration to minimize the interference between the systems, especially, if flexible FDD/TDD boundaries are selected, there may be a need for additional filters in both transmitters and receivers, guard bands that may impact spectrum utilization, and the use of various mitigation techniques for specific situations;

- That selectable/variable duplex technology is considered to be one technique that can assist in the use of multiple frequency bands to facilitate global and convergent solutions. Such a technology could bring further flexibility that would enable IMT terminals to support multiple frequency arrangements;

- When frequency arrangements cannot be harmonized globally, a common base and/or mobile transmit band would facilitate the development of terminal equipment for global roaming. A common base transmit band, in particular, provides the possibility to  roaming users all information necessary to establish a call;

- Guard bands for IMT systems should be minimized to avoid wasting spectrum;

- When developing frequency arrangements, current and future advances in IMT (e.g. multimode/multiband terminals, enhanced filter technology, adaptive antennas, advanced signal processing techniques, techniques associated with cognitive radio systems, variable duplex technology and wireless connectivity peripherals) may facilitate more efficient use and increase overall utilization of radio spectrum; and

- On the aspect of frequency availability it is recommended that administrations make available the necessary frequencies for IMT system development in a timely manner.

- The harmonized frequency arrangements in the bands identified for IMT by one or more conferences shall facilitate:

- facilitate worldwide compatibility; and

- facilitate international roaming.

2.9.1 Frequency arrangements in 700 MHz band

The different recommended frequency arrangements for implementation of IMT in the band 698-960 MHz are summarized below.

A1: 824-849 paired with 869-894 MHz

A2: 880-915   paired with 925-960 MHz

A3: 832-862 paired with 791-821 MHz

A4: 698-716 paired with 728-746 MHz

776-793 paired with 746-763 MHz                with 716-728 MHz unpaired spectrum/TDD

A5: 703-748 paired with 758-803 MHz

A6: 698-806 MHz unpaired spectrum

Due to different usages in 698-960 MHz between regions, – no common solution is possible. In the arrangement A3, reversed duplex direction (mobile transmit in upper band and base transmit in lower band provides better conditions for coexistence with the lower adjacent broadcasting service. In arrangement A4, administrations can use the band solely for FDD or TDD, or some combination of FDD and TDD. Administrations can use any FDD duplex spacing or FDD duplex direction. However, when administrations choose to deploy mixed FDD/TDD channels with a fixed duplex separation for FDD, the duplex separation and duplex direction as shown in A4 are preferred. In A5, 2 x 45 MHz FDD arrangement uses sub blocks with dual duplexer solution and conventional duplex arrangement. Internal guard bands of 5 MHz and 3 MHz are provided at the lower and upper edge of the band for better co-existence with adjacent radio communication services. In A6, taking into account the external 4 MHz guard band (694-698 MHz), a minimum internal guard band of 5 MHz at the lower edge (698 MHz) and 3 MHz at the upper edge (806 MHz) needs to be considered.

2.9.2  Frequency arrangements in the band 1710-2200 MHz:

Different Frequency arrangements in the band 1710-2200 MHz is summarized below:

B1: 1880-1920(TDD), 2010-2025(TDD), 1920-1960(MS Tx) 2110-2170 (BS Tx); 2010-2025 (TDD)

B2: 1710-1785 (MS Tx) and 1805-1880 (BS Tx)

B3: 1850-1910 (MS Tx) and 1930-1990 (BS Tx);  1910-1930 TDD

B4: 1710-1785 (MS Tx) 1805-1880 (BS Tx); 1880-1920 TDD; 1920-1980(MS Tx) 2110-2170(BS Tx); 2010-2025 TDD

B5: 1710-1770 (MS Tx) 2110-2170 (BS Tx); 1850-1910(MS Tx) 1930-1990 ( BS Tx); 1910-1930 TDD

2.9.3 Frequency arrangements in the band 2300-2400 MHz and 2500-2690 MHz:

Frequency arrangements in the band 2300-2400 MHz is shown below:

E1: 2300-2400 TDD

Frequency arrangements in the band 2500-2690 MHz is shown below:

C1: 2500-2570 (MS Tx) 2620-2690 (BS Tx); 2570-2620 TDD

C2: 2500-2570 (MS Tx) 2620-2690 (BS Tx); 2570-2620 (BS Tx external)

C3: Flexible FDD/TDD

Frequency arrangements in the band 3400-3600 MHz:

F1: 3400-3600 TDD/unpaired

F2: 3410-3490 (MS Tx) 3510-3590(MS BS)

3. Some Strategies to be considered to cater the Demand Mobile Broadband

The principle to enable and encourage spectrum to move to its highest value use  or uses- continue review and planning, promote both certainty and flexibility to the extent possible, and investigation to explore potential band for mobile broadband are key aspects to  address the mobile broadband spectrum .

It is very appropriate to use of improved technologies, such as coding, to improve overall spectrum efficiency. Deployment of complementary infrastructure such as picocells and femtocells in dense urban environments, decreases the size of the coverage areas required by individual macro and micro-cells and improves spectrum efficiency. Spectrum demand in highly populated cities need to be investigated in SATRC member countries. Spectrum requirements for mobile broadband as foreseen by the industry over a period by 2020 are applicable to be considered. Harmonized spectrum regional and global level are also the key aspect to be taken into account  for mobile broadband services and applications. It is to be considered what techniques could be deployed in their existing and proposed network architecture to achieve greater spectral efficiencies- advances in technology. Network architecture is very important aspect in mobile broadband -If the cell radius of existing wireless infrastructure was reduced, the demand could be addressed with lesser amount of spectrum. It is needed for analysis of the quantum of spectrum required to meet the demand in SATRC countries. Relocation, Reallocation, Refarming of Spectrum are to be carried out in periodic manner to adopt the change in a timely manner for the benefit of the public. Benefits offered to offload data capacity in small localised environments thus improving data capacity in existing network architecture is another paradigm to be taken into account. Continually reviews bands and planning arrangements to determine whether the band is allocated to its highest value use is also the key principle to be considered.

4 Conclusions

4.1 It is necessary to take into account the following Key considerations in mobile 	broadband shall:

- Facilitate the deployment of innovative broadband services and technologies

- Allocate spectrum to the highest value use or uses

- Promote transparent and nondiscriminatory spectrum management policies

- Embrace technology neutrality

- To adopt flexible use measures for wireless broadband services

- Ensure affordability

- Manage spectrum efficiently

- Harmonize regional and international standards and practices

- Enable and encourage spectrum to move to its highest value use or uses

- To the extent possible, promote both certainty and flexibility

4.2 Frequency arrangements for implementation of IMT

It is recommended to identify the regionally (and globally as far as applicable) harmonized IMT bands capable of delivering quality mobile broadband applications in line with the Recommendation ITU-R M.1036-3 dealing with Frequency arrangements for implementation of the terrestrial component of IMT  in the bands identified for IMT in the Radio Regulations (RR)’ as guidance on the selection of transmitting and receiving frequency arrangements for the terrestrial component of IMT systems. The frequency arrangements shall be used from the point of view of enabling the most effective and efficient use of the spectrum to deliver IMT services – while minimizing the impact on other systems or services in these bands – and facilitating the growth of IMT systems.

The order of the frequency arrangements does not imply any priority. Administrations may implement any of the recommended frequency arrangements to suit their national conditions. The administrations should consider geographical neighbouring deployments as well as matters related to achieving economies of scale, facilitating roaming, and measures to minimize interference.

4.3 The spectrum allocation for the mobile broadband shall be in technology neutral basis to  facilitate for the deployment of effective and efficient technology in the market which will also provide an early upgrade path to take account of more affordable devices including smartphones and tablets in the market.

In order to balance spectrum availability with the legitimate public goal of maximising the price of the country’s spectrum resources, it is recommended that a staged approach to the release of new spectrum be adopted for mobile broadband.

4.4 It is relevant continually to review, and plan for further Mobile Broadband spectrum bands. SATRC countries may consider the aforementioned bands for mobile broadband applications.

4.5 It is recommended to determine the common and harmonised mobile broadband spectrum band  in the SATRC member countries so that it will  :

  - facilitate worldwide compatibility; and

  - facilitate international roaming.

4.6  The principle to enable and encourage spectrum to move to its highest value use  or uses- continue review and planning, promote both certainty and flexibility to the extent possible, and investigation to explore potential band for mobile broadband are key aspects to  address the mobile broadband spectrum . It is very appropriate to use of improved technologies, such as coding, to improve overall spectrum efficiency. Network architecture is very important aspect in mobile broadband -If the cell radius of existing wireless infrastructure was reduced, the demand could be addressed with lesser amount of spectrum

4.7 The lower frequency bands have better building penetration and smaller foliage losses are more cost effective and have larger coverage. It is recommended to make arrangement to deploy IMT advanced technologies can at lower bands and 700 MHz is proving to be most effective for deployment in rural or high-cost regions. Two to three times less sites are required for initial coverage at 700 MHz as compared to 2.1 or 2.5 GHz.

4.8 It is felt that it seems necessary additional information on mobile broadband of the member countries relating to the broadband spectrum requirement. It is agreed to estimate the spectrum requirment in each of the member countries in line with the Questionnaire (prepared and already circulated by the Team Leader of the Report for Spectrum Requirment for Mobile Broadband) by the team members of the WG .Thus it is agreed to provide the reply on the previously structured/send questionnaire as in Annex 1 by the member countries expert. The further analysis would be carried out after receiving the corresponding response to preare final report.

5.  Recommendations

We support the identification of additional frequency bands for IMT to facilitate the development of terrestrial mobile broadband applications.  It is relevant continually to review, and plan for further Mobile Broadband spectrum bands.

1. The exact Spectrum Bandwidth requirment in the SATRC member countries varies from country to country as per the applicable criteria including the market demand, service environment, Radi Environment, demography etc.

2.   It is agreed to estimate dditional spectrum  needed for the year 2020, distributed in 	three band ranges – low band (<1GHz), mid-to-high band (1-3GHz) and high band (3-6GHz), to support the explosive traffic increase and higher performance expectation in the SATRC member countries.  It is agreed that the new spectrum for IMT should be identified mainly below 6 GHz due to technical reasons identified in Report ITU-R M.2074.

- Low bands (< 1GHz) – mainly used for macro network to provide coverage

- Low-to-mid bands (1-3GHz) – mainly used for macro and micro network to  provide coverage/capacity

- Mid-to-high bands (3-6GHz) – mainly use for micro/pico/hotspots network and Wireless Sensor Networks to provide high capacity and performance.

3. The exact requirement of the spectrum in each of the SATRC countries will be contributed by the each WG expert on spectrum based on the ITU spectrum calculation methods using the related attribute parameters and send the same to the Team leader within a month to prepare the final report to enable timely identification of additional IMT spectrum. Further work could be undertaken to investigate the different amounts of spectrum required in urban, suburban and rural areas of the SATRC member countries.

4. Although there are varieties of usage in different IMT candidate bands, SATRC countries may consider the following (but not limited to) bands for mobile broadband applications: 470-694/698 MHz, 694-790 MHz, 1300-1900 MHz, 1980-2010 Mhz paired with 2170-2200 MHz, 1900- 1920/2090-2110 MHz, and 2025-2025/2200-2215 MHz, 2700-2900 MHz, 3400-4200 MHz, 4400-4990 MHz  as far as possible to provide cellular coverage network to harmonize in regional/global basis

700 MHz is very important spectrum which brings significant amount of high quality spectrum for mobile broadband and recommends for the harmonization or compatibility usage for economies of scale and effective/efficient utilization of the band.

Annex-1

QUESTIONNAIRE ON WORK ITEM “Study of regional requirements and availability of spectrum for mobile broadband” - PAKISTAN

Considering the rapid increase in the volume of data traffic in the past few years which is accelerated by the introduction of new technologies and their related applications, this Questionnaire is developed to

- collect the information on the recent usage of mobile communication system in order to see the change of total volume of mobile communication traffic in each country towards the future

- gather information on the mobile market and services expected to be used from around 2012 to 2020

The survey results will be used for the analysis and forecast of services and market aspects which leads to the estimation of the spectrum requirements for future development of mobile broadband and suggestion of harmonized approach to make the spectrum bands available for wireless broadband.

Questions

# Identification of Your Organization

Name of the organization	: Pakistan Telecommunication Authority

Name of contact person		: Ikram Ul Haq Kiani

Email Address 			: ikram@pta.gov.pk

NOTE: You do not necessarily respond to all the questions in this Questionnaire. Please provide any relevant information and your considerations as much as possible.

A. Mobile Band Usage

- Considering that the ITU Radio Regulations identify the bands in the following table as intended for use by administrations wishing to implement IMT, including IMT-2000 and IMT-Advanced,

| Band (MHz) | Footnotes identifying the band for IMT |
| --- | --- |
| 450-470 | 5.286AA |
| 698-960 | 5.312A, 5.313A; 5.317ª |
| 1 710-2 025 | 5.384A, 5.388 |
| 2 110-2 200 | 5.388 |
| 2 300-2 400 | 5.384ª |
| 2 500-2 690 | 5.384ª |
| 3 400-3 600 | 5.430A, 5.432A, 5.432B, 5.433A |

What frequency bands and which cellular technologies are currently used for your mobile network in your country? Please provide detail information, for example, based on the following table format.

| Frequency bands | Currently used Cellular technologies in your mobile network |
| --- | --- |
| UL: X1 MHz ~ X2 MHz DL: Y1 MHZ ~ Y2 MHz | AAA1(*) |
| TDD: Z1 MHz ~ Z2 MHz | AAA2(*) |

(*) Please describe the cellular technologies such as, GSM, CDMA, WCDMA, HSPA, HSPA+, DC-HSPA, LTE(FDD), LTE(TDD), CDMA2000, EV-DO, WiMAX, etc.)

The table below identifies the frequency bands along with technologies that are currently used for mobile/Wireless network in Pakistan.

| Sr.No | Spectrum | Frequency Details | Technology Used |
| --- | --- | --- | --- |
| 1 | 450MHz | UL=452.5-457.6 MHz, DL= 462.5-467.6 MHz (2x 5 MHz BW) | CDMA |
| 2 | 479MHz | UL=479-483.48 MHz, DL= 489-493.48 MHz (2x 4 MHz BW) | CDMA |
| 3 | 1900MHz | UL=1880-1885 MHz , DL= 1960-1965 MHz UL=1890-1900 MHz , DL= 1970-1980 MHz (2x5 MHz Bw) | CDMA 2000,EV-DO |
| 4 | 900MHz | UL=882.5-914.9 MHz, DL= 927.5-959.9 MHz ( ~ 2x35 MHz BW) | GSM |
| 5 | 1800MHz | UL=1710-1745.7 MHz, DL= 1805-1840.7 MHz ( 2x 36 MHz BW) | GSM |
| 6 | 3.5 GHz | 3415.25-3562.25 MHz- Time Division duplex ( 147 MHz BW) | WiMAX |

- Total   = 317 MHZ BW

- Note: Auction for Next Generation Mobile Services/3G/4G 850/1800/2100 MHz band, 30MHz/operator for 6 operators = 180 MHz BW

- Total  Cellular Spectrum BW = 497 MHz

- In addition to the cellular technologies listed in Q1, what kinds of other Radio Access Technologies (RATs) (e.g. Wi-Fi,) are used for your mobile network? Please provide detail information on the RATs and their purposes in your mobile network?

  - Other RATs WiFi

  - Purposes Internet

B. Recent usage of mobile communication system

- What's the total number of subscription per year, estimated traffic volume of mobile communication per year in the unit of Giga Bytes during past five years, including voice and data? If separation on voice and data is possible, please provide the data of each one.

Please provide detail annual information, for example, based on the following table format.

| The end of the year | 2006-07 | 2007-08 | 2008-09 | 2009-10 | 2010-11 |
| --- | --- | --- | --- | --- | --- |
| The number of subscriptions of mobile phone | 62.3 Million | 88 Million | 94.3 Million | 99.2 Million | 100 Million |
| Population estimate | 161.51 Million | 164.44 Million | 167.44 Million | 170.49 Million | 173.59 Million |
| Estimated voice traffic volume |  |  |  |  |  |
| Estimated data traffic volume |  |  |  |  |  |

C. Potential future market aspects and services

C-1. Services and Application

- What are the new services or applications that have gained popularity among large number of users in your mobile network in past few years? If necessary, please provide detail description.

Premium Rate Short Message Services, Location based

- What are the services or applications that generate large amount of mobile data traffic in your mobile network?

SMS and GPRS/EDGE

- What would be the basic services & applications in the future in your mobile network?

Electronic banking

- What would be the expected profitable services and applications in the future in your mobile network?

Electronic banking

# C-2. Developments and Operations

- What are the mobile network factors that you considered the most influencing to decide on if the mobile network should be evolved or not?

  - Coverage				 <please provide details here>

  - Mobile penetration			 <please provide details here>

  - Traffic volume 				 <please provide details here>

  - Service and application			 <please provide details here>

  - Technology and industry progress	 <please provide details here>

  - Others					  <please describe your answer here>

- Do you have a mobile broadband deployment plan of your country?

  - Yes.

  - No.

- If the answer to Q9 is yes, what kind of mobile broadband deployment plans do you have?

- Area coverage by 2020?	   <please describe your answer here>

- Population coverage by 2020?	   <please describe your answer here>

- Available spectrum by 2020? 	   <please describe your answer here>

- Number of forecasted mobile broadband users by 2020? <please describe your answer here>

- Forecasted traffic volume of mobile communication including voice and data up to 2020. If separation on voice and data is possible, please provide the data of each one? <please describe your answer here>

- Others  <please provide details here>

- What are the principles that you take into account during your spectrum planning?

  - Global harmonization

  - Easily to be deployed and used

  - Impact of the services/applications in the adjacent bands

  - Low CAPEX and OPEX

  - Others					 <please describe your answer here>

- What are the industry progress factors that influence your decision on spectrum management and network deployment?

  - Maturity of commercialization network

  - Terminal availability

  - Terminal price

  - Others					 <please describe your answer here>

____________

QUESTIONNAIRE ON WORK ITEM “Study of regional requirements and availability of spectrum for mobile broadband” -Afghanistan

Considering the rapid increase in the volume of data traffic in the past few years which is accelerated by the introduction of new technologies and their related applications, this Questionnaire is developed to

- collect the information on the recent usage of mobile communication system in order to see the change of total volume of mobile communication traffic in each country towards the future

- gather information on the mobile market and services expected to be used from around 2012 to 2020

The survey results will be used for the analysis and forecast of services and market aspects which leads to the estimation of the spectrum requirements for future development of mobile broadband and suggestion of harmonized approach to make the spectrum bands available for wireless broadband.

Questions

# Identification of Your Organization

Name of the organization	: Afghanistan Telecom Regulatory Authority

Name of contact person	: Zabihullah Faizi

Email Address 		: Mohammad  Jan Khan  Watt  Kabul MCIT Building  Afghanistan

NOTE: You do not necessarily respond to all the questions in this Questionnaire. Please provide any relevant information and your considerations as much as possible.

A. Mobile Band Usage

- Considering that the ITU Radio Regulations identify the bands in the following table as intended for use by administrations wishing to implement IMT, including IMT-2000 and IMT-Advanced,

| Band (MHz) | Footnotes identifying the band for IMT |
| --- | --- |
| 450-470 | 5.286AA |
| 698-960 | 5.312A, 5.313A; 5.317ª |
| 1 710-2 025 | 5.384A, 5.388 |
| 2 110-2 200 | 5.388 |
| 2 300-2 400 | 5.384ª |
| 2 500-2 690 | 5.384ª |
| 3 400-3 600 | 5.430A, 5.432A, 5.432B, 5.433A |

What frequency bands and which cellular technologies are currently used for your mobile network in your country? Please provide detail information, for example, based on the following table format.

| Frequency bands | Currently used Cellular technologies in your mobile network |
| --- | --- |
| UL: X1 MHz ~ X2 MHz DL: Y1 MHZ ~ Y2 MHz | AAA1(*) |
| TDD: Z1 MHz ~ Z2 MHz | AAA2(*) |

| Frequency bands | Currently used Cellular technologies in your mobile network |
| --- | --- |
| UL 880-915 MHz DL 925-960 MHz 2x35 MHz BW | E/ GSM 900 |
| UL 1710-1785 MHz DL 1805-1880 MHz 2x75 MHz BW | DCS-1800 |
| UL 824-849 MHz DL 869-894 MHz 2x11 MHz BW | CDMA |
| UL 1920-1980 MHz DL 2110-2170 MHz 2x60 MHz BW | WCDMA,HSPA+ |
| 3300-3600 MHz 300 MHz BW | WiMax |
| 2400-5800 MHz | WiFi |

- Total  Cellular Spectrum=  527 MHz BW

(*) Please describe the cellular technologies such as, GSM, CDMA, WCDMA, HSPA, HSPA+, DC-HSPA, LTE(FDD), LTE(TDD), CDMA2000, EV-DO, WiMAX, etc.)

- In addition to the cellular technologies listed in Q1, what kind of other Radio Access Technologies(RATs) (e.g. WiFi,) are used for your mobile network? Please provide detail information on the RATs and their purposes in your mobile network?

Other RATs 		<please describe your answer here>

Purposes		<please describe your answer here>

B. Recent usage of mobile communication system

- What's the total number of subscription per year, estimated traffic volume of mobile communication per year in the unit of Giga Bytes during past five years, including voice and data. If separation on voice and data is possible, please provide the data of each one.

- Please provide detail annual information, for example, based on the following table format.

| The end of the year | 2007 | 2008 | 2009 | 2010 | 2011 |
| --- | --- | --- | --- | --- | --- |
| The number of subscriptions of mobile phone | 6 Million | 8.5 Million | 12.4 Million | 15.3 Million | 17.6 Million |
| Population estimate | 19% | 28.4% | 41.3% | 51% | 58.6% |
| Estimated voice traffic volume |  |  |  |  |  |
| Estimated data traffic volume |  |  |  |  |  |

C. Potential future market aspects and services

C-1. Services and Application

- What are the new services or applications that have gained popularity among large number of users in your mobile network in past few years? If necessary, please provide detail description.

- Answer: internet Access,SMS,remote recharging

- What are the services or applications that generate large amount of mobile data traffic in your mobile network?

Answer: internet Access and WAP

- What would be the basic services & applications in the future in your mobile network?

- Answer: electronic payment and broadband internet

- What would be the expected profitable services and applications in the future in your mobile network?

- Answer: internet Access and Mobile voice

# C-2. Developments and Operations

- What are the mobile network factors that you considered the most influencing to decide on if the mobile network should be evolved or not?

  - Coverage				 <please provide details here>

  - Mobile penetration			 <please provide details here>

  - Traffic volume 				 <please provide details here>

  - Service and application			 <please provide details here>

  - Technology and industry progress		 <please provide details here>

  - Others			  <please describe your answer here>

- Do you have a mobile broadband deployment plan of your country?

  - Yes.

  - No.

- If the answer to Q9 is Yes, what kind of mobile broadband deployment plans do you have?

- Area coverage by by 2020?	   <please describe your answer here>

- Population coverage by 2020?	   <please describe your answer here>

- Available spectrum by 2020? 	   <please describe your answer here>

- Number of forecasted mobile broadband users by 2020? <please describe your answer here>

- Forecasted traffic volume of mobile communication including voice and data up to 2020. If separation on voice and data is possible, please provide the data of each one? <please describe your answer here>

- Others  <please provide details here>

- What are the principles that you take into account during your spectrum planning?

  - Global harmonization

  - Easily to be deployed and used

  - Impact of the services/applications in the adjacent bands

  - Low CAPEX and OPEX

  - Others					 <please describe your answer here>

- What are the industry progress factors that influence your decision on spectrum management and network deployment?

  - Maturity of commercialization network

  - Terminal availability

  - Terminal price

  - Others					 <please describe your answer here>

____________

Response on QUESTIONNAIRE ON WORK ITEM “Study of regional requirements and availability of spectrum for mobile broadband”- Nepal

Considering the rapid increase in the volume of data traffic in the past few years which is accelerated by the introduction of new technologies and their related applications, this Questionnaire is developed to

- collect the information on the recent usage of mobile communication system in order to see the change of total volume of mobile communication traffic in each country towards the future

- gather information on the mobile market and services expected to be used from around 2012 to 2020

The survey results will be used for the analysis and forecast of services and market aspects which leads to the estimation of the spectrum requirements for future development of mobile broadband and suggestion of harmonized approach to make the spectrum bands available for wireless broadband.

Questions

# Identification of Your Organization

Name of the organization	: Nepal Telecommunications Authority

Name of contact person	:  Min Prasad Aryal

Email Address 		: <please type your answer here>

NOTE: You do not necessarily respond to all the questions in this Questionnaire. Please provide any relevant information and your considerations as much as possible.

A. Mobile Band Usage

- Considering that the ITU Radio Regulations identify the bands in the following table as intended for use by administrations wishing to implement IMT, including IMT-2000 and IMT-Advanced,

| Band (MHz) | Footnotes identifying the band for IMT |
| --- | --- |
| 450-470 | 5.286AA |
| 698-960 | 5.312A, 5.313A; 5.317ª |
| 1 710-2 025 | 5.384A, 5.388 |
| 2 110-2 200 | 5.388 |
| 2 300-2 400 | 5.384ª |
| 2 500-2 690 | 5.384ª |
| 3 400-3 600 | 5.430A, 5.432A, 5.432B, 5.433A |

What frequency bands and which cellular technologies are currently used for your mobile network in your country? Please provide detail information, for example, based on the following table format.

CDMA800:   824-844 MHz paired with 869-879 MHz                             2x10 MHz BW

GSM 900:     880-915MHz paired with 925-960MHz                               2x35 MHz BW

GSM 1800:  1710-1785MHz paired with 1805-1880MHz		 2x75MHzBW               3G/IMT-2000: 1920-1980MHz paired with 2110-2170MHz		2x60 MHz BW

BWA/IMT-Advanced:  2300-2400 MHz unpaired   	                       100 MHz BW

BWA/IMT-Advanced: 2500-2690 MHz                    	        		190 MHz

BWA/WiMAX/IMT Advanced:    3300-3400 MHz	     		100 MHz

IMT Advanced/LTE:     700   MHz 		     				108 MHz

Total  Cellualr Spectrum = 858 MHz BW

- In addition to the cellular technologies listed in Q1, what kind of other Radio Access Technologies(RATs) (e.g. WiFi,) are used for your mobile network? Please provide detail information on the RATs and their purposes in your mobile network?

- Other RATs 		WiFi, VSAT

- Purposes		 Internet, Link

B. Recent usage of mobile communication system

- What's the total number of subscription per year, estimated traffic volume of mobile communication per year in the unit of Giga Bytes during past five years, including voice and data. If separation on voice and data is possible, please provide the data of each one.

- Please provide detail annual information, for example, based on the following table format.

| The end of the year | 2009 | 2010 | 2011 |
| --- | --- | --- | --- |
| The number of subscriptions of mobile phone | 6,387,196 | 10,041,499 | 1,33,54,477 |
| Population estimate | 2,75,04,280 | 28,584,975 | 26,620,809 |
| Estimated voice traffic volume |  |  |  |
| Estimated data traffic volume |  |  |  |

C. Potential future market aspects and services

C-1. Services and Application

- What are the new services or applications that have gained popularity among large number of users in your mobile network in past few years? If necessary, please provide detail description.

- voice, internet (GPRS, EDGE,3G based)

- What are the services or applications that generate large amount of mobile data traffic in your mobile network?

- Voice, Internet

- What would be the basic services & applications in the future in your mobile network?

- Mobile broadband applications, e-applications

- What would be the expected profitable services and applications in the future in your mobile network?

- Mobile broadband applications, e- applications (e.g. e-governance, e-banking, e-commerce)

# C-2. Developments and Operations

- What are the mobile network factors that you considered the most influencing to decide on if the mobile network should be evolved or not?

  - Coverage				√ <please provide details here>

  - Mobile penetration			√ <please provide details here>

  - Traffic volume 				√ <please provide details here>

  - Service and application			√ <please provide details here>

  - Technology and industry progress	√ <please provide details here>

  - Others					    <please describe your answer here>

- Do you have a mobile broadband deployment plan of your country?

  - Yes.

  - No.		√

- If the answer to Q9 is Yes, what kind of mobile broadband deployment plans do you have?

- Area coverage by by 2020?	   <please describe your answer here>

- Population coverage by 2020?	   <please describe your answer here>

- Available spectrum by 2020? 	   <please describe your answer here>

- Number of forecasted mobile broadband users by 2020? <please describe your answer here>

- Forecasted traffic volume of mobile communication including voice and data up to 2020. If separation on voice and data is possible, please provide the data of each one? <please describe your answer here>

- Others  <please provide details here>

- What are the principles that you take into account during your spectrum planning?

  - Global harmonization					√

  - Easily to be deployed and used				√

  - Impact of the services/applications in the adjacent bands	√

  - Low CAPEX and OPEX					√

  - Others					 <please describe your answer here>

- What are the industry progress factors that influence your decision on spectrum management and network deployment?

  - Maturity of commercialization network	√

  - Terminal availability			√

  - Terminal price				√

  - Others					 <please describe your answer here>

____________

New Questionnaires Reply

SRI LANKA

# QUESTIONNAIRE REGARDING requirements and availability of spectrum for mobile broadband

Considering the rapid increase in the volume of data traffic in the past few years which is accelerated by the introduction of new technologies and their related applications, this Questionnaire is developed to collect the information on the recent usage of mobile communication system in order to see the change of total volume of mobile communication traffic in each SATRC country towards the future and to gather information on the mobile market and services expected to be used from around 2014 to 2020 for the analysis and forecast of services and market aspects which leads to the estimation of the spectrum requirements for future development of mobile broadband and suggestion of harmonized approach to make the spectrum bands available.

- What's the total number of subscription per year, traffic volume of mobile communication per year in the unit of Giga Bytes during past five years, including voice and data? If separation on voice and data is possible, please provide the data of each one. Please provide detail annual information, for example, based on the following table format.  Elaborate the Subscriptions with Mobile Subscriber Growth Rate (with Pictural Presentation-Bar Chart) since 2008 to 2013.

| The end of the year |  | 2008 | 2009 | 2010 | 2011 | 2012 | 2013 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| The number of subscriptions of mobile phone |  | 11082454 | 14264442 | 17267407 | 18319447 | 20234070 | 20315150 |
| Population estimate |  |  | 20450000 | 20653000 | 20869000 | 20277597 | 20480000 |
| voice traffic volume (Billion) in Minutes |  |  |  |  |  | 43.3 | 52.5 |
| Data traffic volume |  |  |  |  |  | N/A | N/A |

- Estimate the traffic volume of mobile communication per year in the unit of Giga Bytes including voice and data till 2020 as per the followings? Elaborate the Subscriptions with Mobile Subscriber Growth Rate (with Pictural Presentation-Bar Chart) since 2015 to 2020.

|  | 2015 | 2016 | 2017 | 2018 | 2019 | 2020 |
| --- | --- | --- | --- | --- | --- | --- |
| The number of subscriptions of mobile phone |  |  |  |  |  |  |
| Population estimate |  |  |  |  |  |  |
| voice traffic volume (Billion) |  |  |  |  |  |  |
| Data traffic volume |  |  |  |  |  |  |

- What are the new services or applications that have gained popularity among large number of users in your mobile network in past few years? If necessary, please provide detail description. (e.g. Mobile Internet Applications (e.g. Facebook, Twitter, email, Voice, Internet access, games, e-commerce, remote education, telemedicine, multimedia applications) Mobile Internet, Mobile Banking, Mobile Cash Exchange, Online Gamming

- What are the services or applications that generate large amount of mobile data traffic in your mobile network? Mobile Internet, Online Gamming

- What would be the basic services & applications in the future in your mobile network? Voice, Internet

- What would be the expected profitable services and applications in the future in your mobile network? Internet

- Provide the number of 2G, 3G, and 4G Base Stations since 2008 to 2013. Also provide your estimation/forecast for Network 2G, 3G and 4G Base stations upto 2020. Exact details are not available

8.  What would be the spectrum requirement for the deployment of 2G, 3G, and 4G networks till 2020 in your country? Use appropriate/rational Model for the Calculation of spectrum with annual growth.

9. What would be the technical process of estimating the spectrum requirements for 	mobile communications? Elaborate in respect of Definition of services, Market 	expectations, Technical and operational framework, and Spectrum calculation 	method.

10.	What would be the estimated voice traffic and data traffic in the Geotypes- Rural, sub-urban, and urban areas for Spectrum BW Required for 2G, 3G & 4G spectrum?

11. Do you have specific plan for launching of 4G network. If yes provide the scheduled plan for the deployment. Already Deployed LTE in 1.8G, 2.3G and 2.6G bands

12. What are some of the emerging technologies that providers may consider deploying in their respective network architecture in the next  8 years i.e by 2020 ? Advanced IMT Technologies

13. What type of Radio environment (RE) do you explore in the future upto 2020 in your country? Note that REs are defined by the cell layers in a network consisting of hierarchical cell layers, i.e. macro, micro, pico and hot-spot cells. Depend on the requirement and coverage

14. Point out the different existing spectrum bands/bandwidth those are available for mobile celular technologies in your country. Mention with the lower and upper frequency along with the corresponding BW.

880-915MHz paired with 925-960MHz

1710-1785MHz paired with 1805-1880MHz

1905-1920MHz unpaired

1920-1980MHz paired with 2110-2170MHz

15.  Explore the possible spectrum bands/BW for the celluar mobile technologies to be availed to cater the market demand by 2020 in your country. Point out the possible bands along with the BW.

698-806MHz

16. What are the mobile network factors that you considered the most influencing to decide on if the mobile network should be evolved or not?

  - Coverage				X <please provide details here>

  - Mobile penetration			X <please provide details here>

  - Traffic volume 			X <please provide details here>

  - Service and application		X <please describe your answer here>

  - Technology and industry progress	X <please provide details here>

  - Others					  <please describe your answer here>

- 17.	Do you have a mobile broadband deployment plan?

  - Yes.		X

  - No.

- 18. what kind of mobile broadband deployment plans do you have?

- Area coverage by 2020?	   <please describe your answer here>

- Population coverage by 2020?	Increase broadband penitration more than 60%

- Available spectrum by 2020? 	   Availability of 700MHz band

- Number of forecasted mobile broadband users by 2020? 05

- Forecasted traffic volume of mobile communication including voice and data up to 2020. If separation on voice and data is possible, please provide the data of each one? <please describe your answer here>

- Others  <please provide details here>

- 19.	What are the principles that you take into account during your spectrum planning?

  - Global harmonization					X

  - Easily to be deployed and used

  - Impact of the services/applications in the adjacent bands	X

  - Low CAPEX and OPEX

  - Others					 <please describe your answer here>

- 20. What are the factors that influence your decision on spectrum management and  	network deployment?

  - Maturity of commercialization network

  - Terminal availability

  - Terminal price

  - Others					 <please describe your answer here>

- 21.	Provide any other relevant issues regarding the spectrum requirment for mobile broadband.

Nepal

# QUESTIONNAIRE REGARDING requirements and availability of spectrum for mobile broadband

Considering the rapid increase in the volume of data traffic in the past few years which is accelerated by the introduction of new technologies and their related applications, this Questionnaire is developed to collect the information on the recent usage of mobile communication system in order to see the change of total volume of mobile communication traffic in each SATRC country towards the future and to gather information on the mobile market and services expected to be used from around 2014 to 2020 for the analysis and forecast of services and market aspects which leads to the estimation of the spectrum requirements for future development of mobile broadband and suggestion of harmonized approach to make the spectrum bands available.

      - What's the total number of subscription per year, traffic volume of mobile communication per year in the unit of Giga Bytes during past five years, including voice and data? If separation on voice and data is possible, please provide the data of each one. Please provide detail annual information, for example, based on the following table format.  Elaborate the Subscriptions with Mobile Subscriber Growth Rate (with Pictural Presentation-Bar Chart) since 2008 to 2013.

| The end of the year |  | 2008 | 2009 | 2010 | 2011 | 2012 | 2013 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| The number of subscriptions of mobile phone |  |  | 6,387,196 | 10,041,499 | 1,33,54,477 |  |  |
| Population estimate |  |  |  |  |  |  |  |
| voice traffic volume (Billion) in Minutes |  |  |  |  |  |  |  |
| Data traffic volume |  |  |  |  |  |  |  |

- Estimate the traffic volume of mobile communication per year in the unit of Giga Bytes including voice and data till 2020 as per the followings? Elaborate the Subscriptions with Mobile Subscriber Growth Rate (with Pictural Presentation-Bar Chart) since 2015 to 2020.

|  | 2015 | 2016 | 2017 | 2018 | 2019 | 2020 |
| --- | --- | --- | --- | --- | --- | --- |
| The number of subscriptions of mobile phone |  |  |  |  |  |  |
| Population estimate |  |  |  |  |  |  |
| voice traffic volume (Billion) |  |  |  |  |  |  |
| Data traffic volume |  |  |  |  |  |  |

- What are the new services or applications that have gained popularity among large number of users in your mobile network in past few years? If necessary, please provide detail description. (e.g. Mobile Internet Applications (e.g. Facebook, Twitter, email, Voice, Internet access, games, e-commerce, remote education, telemedicine, multimedia applications)

Mobile Internet, Mobile Banking, Mobile Cash Exchange, Online Gaming, Social Packs ( Facebook, Twitter, Linkedin),Email (Outlook, Gmail),Chat (Facebook, Gmail, Mig33, Yahoo, MSN ), Messaging services (Whatsapp, Line, Viber, Wechat, Skype),News (ekantipur, Onlinekhabar), Streaming( Youtube, Daily Motion),Online TV( Starsports, Starplus), Online Games( Temple Run, Subway surf, Candy Crush), 2P based services (Torrent),Movie and songs downloads

- What are the services or applications that generate large amount of mobile data traffic in your mobile network?

- Mobile Internet,

- Online Gaming,

- Social Packs

- streaming,

- News based services

- URL based services (Facebook pack)

- IPTV (Mobile TV)

- Fiber to your home

- Ecommerce

- Video conferencing

- Online Games

- Messaging services

- Cloud based services

- Machine2Machine

- Telemedicine

- Mobile Money

- Streaming services

- Remote Education

- Chat based services

- Email services

- News based services

- What would be the expected profitable services and applications in the future in your mobile network?

- IPTV

- Messaging services

- Telemedicine

- Remote Education

- Video Conferencing

- Streaming services

- Mobile Money

- E-commerce

- Mobile Advertising

- Provide the number of 2G, 3G, and 4G Base Stations since 2008 to 2013. Also provide your estimation/forecast for Network 2G, 3G and 4G Base stations upto 2020. Exact details are not available

- What would be the spectrum requirement for the deployment of 2G, 3G, and 4G networks till 2020 in your country? Use appropriate/rational Model for the Calculation of spectrum with annual growth.

Not Available

- What would be the technical process of estimating the spectrum requirements for 	mobile communications? Elaborate in respect of Definition of services, Market expectations, Technical and operational framework, and Spectrum calculation method.

It is necessary to take into account the demand of services, effective and efficient technical and operational framework with optimal spectral efficiency. The calculation has to consider the number of base stations within a given market and scope for future site densification. The spectrum calculation model has also has to consider the traffic load per site applicable for both the data and voice.

- What would be the estimated voice traffic and data traffic in the Geotypes- Rural, sub-urban, and urban areas for Spectrum BW Required for 2G, 3G & 4G spectrums?

Details not available

- Do you have specific plan for launching of 4G network. If yes provide the scheduled plan for the deployment.

Not defined yet, however the band 700 MHz and 2600 MHz is identified for the LTE

- What are some of the emerging technologies that providers may consider deploying in their respective network architecture in the next 8 years i.e by 2020?

Considering the mobile technology growth in emerging market as well as the developed market, the same evolution path, i.e. evolving from 2G voice centric to 3G, LTE/ data centric. Apart from that with data boom, the data offloading via Wifi technology might be the emerging technology.

- What type of Radio environment (RE) do you explore in the future upto 2020 in your country? Note that REs are defined by the cell layers in a network consisting of hierarchical cell layers, i.e. macro, micro, pico and hot-spot cells.

Depend on the requirement and coverage

Considering the data demand, deployment of the hierarchical cell layer with micro, Pico cells to serve the enterprise client to provide better indoor network as most of the data are coming from indoors.

13.    Point out the different existing spectrum bands/bandwidth those are available for mobile cellular technologies in your country. Mention with the lower and upper frequency along with the corresponding BW.

824-844 MHz paired with 869-879 MHz: CDMA800                   2x10 MHz BW

880-915MHz paired with 925-960MHz: GSM 900                        2x35 MHz BW

1710-1785MHz paired with 1805-1880MHz: GSM 1800              2x75 MHz BW

1920-1980MHz paired with 2110-2170MHz	: 3G/IMT-2000        2x60 MHz BW

2300-2400 MHz unpaired:   	 BWA/IMT-Advanced                   100 MHz BW

2500-2690 MHz               :       BWA/IMT-Advanced	        190 MHz

3300-3400 MHz	     :	BWA/WiMAX/IMT Advanced     100 MHz

700   MHz 		     :     IMT Advanced/LTE		        ………..

14. Explore the possible spectrum bands/BW for the celluar mobile technologies to be availed   to cater the market demand by 2020 in your country. Point out the possible bands along with the BW.

450 MHz/700 MHz/ 2700 MHz/ 3400-4200 MHz/ 4400-4900 MHz

15. What are the mobile network factors that you considered the most influencing to decide on if the mobile network should be evolved or not?

- Coverage				X <please provide details here>

- Mobile penetration			X <please provide details here>

- Traffic volume 			X <please provide details here>

- Service and application		X <please describe your answer here>

- Technology and industry progress	X <please provide details here>

- Others					  <please describe your answer here>

- 16.	Do you have a mobile broadband deployment plan?

- Yes.

- No.		X  No Specific Deployment Plan , however the 3G/4G licensing would    be carried out

- 17. what kind of mobile broadband deployment plans do you have?

- Area coverage by 2020?	   <please describe your answer here>

- Population coverage by 2020?	Increase broadband penitration more than 60%

- Available spectrum by 2020? 	   Availability of 700MHz band

- Number of forecasted mobile broadband users by 2020? 05

- Forecasted traffic volume of mobile communication including voice and data up to 2020. If separation on voice and data is possible, please provide the data of each one? <please describe your answer here>

- Others  <please provide details here>

- 18.	What are the principles that you take into account during your spectrum planning?

- Global harmonization					X

- Easily to be deployed and used			 X

- Impact of the services/applications in the adjacent bands	X

- Low CAPEX and OPEX

- Others					 <please describe your answer here>

- 19. What are the factors that influence your decision on spectrum management and  	network deployment?

- Maturity of commercialization network	X

- Terminal availability				X

- Terminal price					X

- Others					 <please describe your answer here>

- 20. Provide any other relevant issues regarding the spectrum requirment for mobile broadband.

India

It has envisaged the strategy to make available adequate globally harmonized IMT spectrum in 450 MHz, 700 MHz, 1800 MHz, 1910 MHz, 2.1 GHz, 2.3 GHz, 2.5 GHz, 3.5 GHz bands and other bands to be identified by ITU for commercial mobile services.

Recognizing this growth of IMT services and considering the future requirements, the National Telecom Policy 2012 of India highlights the need of adequate availability of spectrum with a target of making available additional 300 MHz spectrum for IMT services by the year 2017 and another 200 MHz by 2020.

PMSE: Program Making & Special Events

Source: Revised spectrum forecasts using the new spectrum model Spectrum required for various mobile communications markets in 2020 prepared for GSMA Jan 2013 by Coleago Consulting